# Lab 6 manual: Linear combination of resistors

Dr. D. Beznosko

5/1/2026

For PHYS 1112L only

(Lab outcomes: Solving a linear system of equations using computer. Reminder – do not include this part with your lab report!)

## Abstract

The measurement of even a simple quantity like resistance can be difficult at times as it’s not always possible to separate the parts of the circuit for this measurement. The report describes the procedure of measuring the resistance of the inseparable circuit and suggests other possibilities for the measurements. Linear system of equations needs to be solved, and errors propagated.

## Introduction

The solutions for the systems of equations are often needed in numerous practical applications. The most common example of linear system solutions is the eigenvalue problem that is widely used in physics, engineering, and other sciences. It is at the base of machine learning. Can you find any more examples and practical uses?

## Experimental Setup

The experimental setup in Figure 1 shows the three resistors connected as a triangle. The resistance can be measured by the multimeter between the points A, B and C only.

R1

C

R2

R3

A

B

Multimeter

Figure 1: The experimental setup schematic. A multimeter is used to measure the resistance between the points A, B and C.

### Procedure

Using multimeter, measure RAB, RBC and RCA. Using the manual for the multimeter, get the accuracy and use it as systematic error. What is the statistical error? (Hint – if the digits don’t change, its half of the smallest digit. Otherwise, do the 5 measurements of one of the three resistances and use the stdev as statistical error).

In the theory section, write the expressions for RAB, RBC and RCA in terms of R1, R2 and R3. Can be left as reciprocal relationships, that is, 1RAB  ,1RBC and1RCA.

## Experimental Data

The results of the measurements should be organized as Table 1 or similar. Think about how to organize this table. The error can be listed in a different column or as value ± error unit. Don’t include all data values here, only relevant means, stdev etc. All additional data must be included in the Appendix for this lab. In most cases, all data points are not needed, especially for computer-based labs.

Provide description about each entry as needed, don’t just leave the table without text in this section. Organize well so that data takes up less space, but everything needed is included.

Table 1: Experimental data.

| Quantity name | Measured value, units |
| --- | --- |
| R Measurement | Resistance, Ω |
| RAB |  |
|  | Errors! |

## Analysis

Use the Wolfram Alpha [2] or similar computer software/online service to solve the system of equations for the R1, R2 and R3. Propagate errors and compare with the values of the resistors from the color marking. The marking has the %error, so each value here is also with error. Do they match?

## Conclusion

Comment on your result (that is, R1, R2 and R3) and the methods used to find it. What did you learn in performing this measurement? Where else can it be used? Can it be improved?

# References

| [1] | PASCO Scientific, 10101 Foothills Boulevard, Roseville, California 95747 USA. |
| --- | --- |
| [2] | "Wolfram Alpha," Wolrfam Inc., [Online]. Available: https://www.wolframalpha.com/. |

Sample command: Solve[{1/r12 == 1/r1 + 1/(r2 + r3), 1/r23 == 1/r2 + 1/(r1 + r3),    1/r31 == 1/r3 + 1/(r2 + r1)}, {r1, r2, r3}], here r12 is for RAB, etc.