# Lab 3 manual: Capacitors

Dr. D. Beznosko

7/31/2026

For PHYS 1112L only

(Lab outcomes: multiple measurements across physical system, linearization of the reciprocal fit and error bars handling. Using the web-based fitting tool https://sos.clayton.edu/physics/chi2/ or a backup at http://www.physicsplot.com/ )

## Abstract

A capacitor is a device to store charge and energy in the form of the electric field. In this lab, we will explore the dependance of the main property of the capacitor – its capacitance – on the distance between the plates in the simplest geometric case. Conversion of the error bars when trying to linearize a plot will also be explored.

## Introduction

Leiden jars were the first capacitors. Read more on the history, it’s exciting. There are many types of capacitors from air to foil to electrolytic and ceramic. Even the integrated circuits have tiny capacitors. Tiny in size, but what about their capacitance? A capacitor can be used in a large variety of circuits from the rectifiers to multiple uses in the AC circuits. Can you find a few of those? Where else capacitors can be used? Can we use them to power cars?

## Experimental Setup

Multimeter with capacitance input

Metal Plates

Index cards

The setup in Figure 1 consists of paper index cards, two metal places and a multimeter with a capacitor probe input.

Figure 1: The experimental setup schematic.  Two metal places and paper index cards are used to produce a capacitor. Connection wires are not shown.

### Procedure

The goal of the experiment is to find the dependance of capacitance on geometric parameters and the dielectric coefficient of the paper. For that, simple measurements will be performed as follows:

Stack all the index cards together. There should be 30 cards. Using caliper, measure the stack thickness at least at 10 different places. Don’t compress the paper hard when doing this. Find the total thickness of a single card with error (hint – remember what happens with error when divided or multiplied by a constant).

Measure the width and heigh of the metal plates (we don’t need the thickness). Measure each at least 10 times along each dimension and include both plates into the measurement. Write all errors down and calculate the area of the plates. Remember to propagate errors when needed.

Place 5 cards between the metal planes and fix them together. Make sure that the holding devices (typically, clothes pins) are not shorting the plates with any metal parts they might have. Measure the capacitance. Move the connecting wires, measure again a few times. Remove the cards and separate the plates with maximum distance that the wires allow and measure again. What does this tell you about the measurement errors? Check the multimeter manual for the precision and use it as part of the error. Don’t separate systematic and statistical errors but combine in quadrature into a dingle error.

Measure capacitance for 10, 15 … 30 cards. You should end up with 6 data points.

## Experimental Data

The results of the measurements should be organized as Table 1 or similar. Think about how to organize this table. The error can be listed in a different column or as value ± error unit. Don’t include all data values here (like the same value measured 5 times), only relevant means, stdev etc. In most cases, individual data points are not needed, especially for computer-based labs. What are the distance and capacitance errors?

Table 1: Experimental data. Include all 6 data points here.

| D, m | ΔD, m | C, nF | ΔC, nF |
| --- | --- | --- | --- |

Provide description about each entry as needed, don’t just leave the table without text in this section. Organize well so that data takes up less space, but everything needed is included. Explain in the section text the meaning of the column headers.

## Theory

The relationship between the capacitance and the geometry of the plates is given by the following equations:

C0=ε0Ad1

where C0 is the capacitance without any material between plates, ε0 is the permittivity of free space (8.8542∙10−12C2Nm2, this is for your information but avoid writing the values of commonly known constants in your report), A is the surface area of a plate of the capacitor, and d is the separation distance between the plates. If the space between the plates is filled with anything but vacuum, the equation changes:

C=κε0Ad2

with κ being the dielectric constant for the material. Note that for the air, κ≈1.0006 so we don’t consider air to be different from vacuum within our measurement precision.

Additional equations will be needed, like error propagations, meaning for the slope and how to get κ and its error from the slope, etc.

## Analysis

The analysis will have two plots. Show supporting calculations, etc.

For the first one, use equation 2. Plot the measured capacitance vs. the reciprocal distance between the plates. Think about how the error for the distance will change here? Hint: 1d=f and you are looking for Δf given Δd. Simplified propagation equations can be used here. Do the linear fit, find the quality of the fit, that is χ2ndf, and convert it into the probability using http://courses.atlas.illinois.edu/spring2016/STAT/STAT200/pchisq.html. Use the slope (with error of course) and use it to find κ and its error.

From the second plot, we want to get the dielectric constant for the paper from the plot directly as a slope. Take the ratio of eq. 2 over eq. 1 and show that κ will equal to the slope. Plot the measured capacitance vs. the calculated one from area and distances using equation 1. The slope of the line is what you seek. (This is a plot, what other things should be done? Remember to label the axes, note the units. As you are including captions with every figure, there is no need for the figure title.) Try to find the dielectric constant of paper from various sources and compare with your result. Why is it not matching well? Is that only measurement errors or something else can be interfering? Hint – paper is not a name for a single material. Compare the results from both plots you have and the accepted value.

## Conclusion

Comment on your result and the methods used to find it. What did you learn in performing this measurement? Where else can it be used? Can this experiment be improved?

# References

| [1] | PASCO Scientific, 10101 Foothills Boulevard, Roseville, California 95747 USA. |
| --- | --- |