# Lab 4 manual: Resistivity and Linearity of Ohm’s Law

Dr. D. Beznosko

7/2/2026

For PHYS 1112L only

(Lab outcomes: Instrumental error; linear mapping measurement, usage of x and y error bars for χ2ndf)

## Abstract

This document describes the measurement of material resistivity using a linear mapping measurement process on a long thin wire. The use of both x- and y-error bars and the fit quality is introduced. Remember to include the result in the abstract of your report.

## Introduction

Figure 1: The experimental setup consists of Wheatstone bridge apparatus and multimeter.

Multimeter

Wheatstone bridge apparatus

Resistance to the electric current is the property of the actual circuit element in its shape and form and it includes not only the property of the material it’s made of, but the geometric factor as well. The property of bulk material is called resistivity ρ – do not confuse with density! There is also conductivity with is the inverse of resistivity, thus it’s not an independent property and it is sufficient to measure just one. To measure resistivity, the geometric factor should be accounted for. Think about Pouillet's law may be? Where and how in life resistance is important? Are there any devices that use resistance for its operations?

## Experimental Setup

The experimental setup is shown in Figure 1 and contains the Wheatstone bridge apparatus and an ohmmeter (or a multimeter). Only the resistive wire and probe are used.

### Procedure

First, use the caliper to measure the diameter of the wire. Do this at least at 10 different positions along the ruler. You will use the average and stdev. What about systematic error?

Before you start, you need to choose the right setting for the multimeter. The lower is the setting, the better is the precision and accuracy. Choose the setting such as not to change it during the measurement (preferred here, if entire range can fit into the same setting. In some cases, the change will be preferred but not in this setup). Look into the multimeter manual to figure out the instrument accuracy for the chosen setting and use it as systematic error.

What other systematic error is there? Think about the connecting wires. Do they have resistance? Is it small compared to values that are being measured? Check what this resistance is. Explain why we don’t care about this value in the report (hint – related to plot and offset?)

Connect the multimeter as in Figure 1. Place the probe about half-way around the 50cm mark. Record the resistance. Move the probe, shake the wire carefully and repeat this measurement at the same position for 4 more times. Stdev is the statistical error. You will combine errors into one as statisticalerror²+systematicerror² as we will use this in a plot.

Now think about the error for the distance and the thickness of the probe. And the wire motion, as it is not fixed to the apparatus tightly. What should you use as the distance error? We assume that ruler accuracy is much better than our use of it, so we do not consider systematic error here, only statistical.

Do at least 9 measurements keeping them equally spaced along the wire (i.e., the linear map). While equal spacing is not a requirement, it helps with the overall accuracy as it reduces the contribution of errors by keeping the differences between the measurements as large as possible (you should not measure the 0 cm and should skip 10 cm). Record Ri±ΔRi. Remember that error changes for each measurement as accuracy is different! Does it affect the measurement much? Can the same error be used instead for all points?

## Experimental Data

The results of the measurements should be organized similarly to Table 1. Also, for the voltage and current. Think about how to organize the tables. The error can be listed in a different column or as value ± error unit. Use SI units! May be this should be in the appendix?

Table 1: Experimental data for which field plate?

| D, m | R, Ω |
| --- | --- |
|  | Use different columns for error or include them with the values |

Provide description about each entry as needed, don’t just leave the table without text in this section. Organize well so that data takes up less space, but everything needed is included.

## Theory

Consider equation 1 that shows the expression for the resistivity:

ρ=RAl1

with R being the resistance of the wire segment, and A∧l – area and length of the segment. Always use equation editor and number the equations so that they later can be referenced as eq. 1 or equation 1. Other equations may need to be used for this lab. Show any error propagation equations as well! Here you need to use at least one!

Analysis

For analysis, we will use Excel or similar spreadsheet software. If you use any professional software, that would be ok, but it must support the necessary features.

Part 1:

We want the slope to be resistivity – what should be the axes? Figure out the total errors for the error bars. Do the plot. If errors bars are not visible, mention this in text. (hint – your Y values may be too small, may be choosing slope to be ρA is going to work better for the plot).

Do the linear fit using pol1 function, find the resistivity with error. Use https://sos.clayton.edu/physics/chi2/

## Conclusion

Remember to include references and appendices as needed. Comment on your result. Try to figure out the material of the wire from the value of resistivity. What did you learn in performing this measurement? Where else can it be used? Can it be improved?

# Bibliography

| [1] | PASCO Scientific, 10101 Foothills Boulevard, Roseville, California 95747 USA. |
| --- | --- |
| [2] | "Chi^2 to P calculator," [Online]. Available: http://courses.atlas.illinois.edu/spring2016/STAT/STAT200/pchisq.html. |