# Lab 10 manual: Index of refraction of water

Dr. D. Beznosko

8/19/2022

For PHYS 1112L and PHYS 2212L

(Lab outcomes: measuring angles and refraction index. Error propagation application to angles. Laser safety.)

## Abstract

The Snell’s law is used to measure the refraction index of water. Laser safety is explained, and goniometer is introduced. The angle errors are propagated through all equations.

## Introduction

The refraction n is the property of all materials, even non-transparent – in general, n is complex, and the imaginary part is the absorption, so non-transparent media will have only the imaginary part. Can n be less than 1 or negative? Can you find any more examples and practical uses?

## Experimental Setup

Figure 1: The experimental setup schematic.

Screen

Goniometer

Laser

Water reservoir with glass walls

Light path without water

Light path with water

Distance between walls

The experimental setup in Figure 1 shows the glass reservoir for water, the laser, and the goniometer. A piece of sticky tape is used for the screen.

### Procedure

Listen to laser safety very carefully. Never look at the laser directly (the light emission point), never point it at any person or reflective surface!!

Fill the apparatus with water. Place the sticky tape on the reservoir wall. Do the measurements at 0o−60o. Mark all laser beam positions on the tape. Make sure that you aim the laser correctly (instructor will demonstrate). What is the error on θair? Measure distances of all points on the take from the 0-angle point. What is the error on these measurements? Using caliper, measure the distance between the glass walls as indicated in Figure 1. Consider this error as well.

## Experimental Data

The results of the measurements should be organized as Table 1 or similar. Think about how to organize this table. The error can be listed in a different column or as value ± error unit. Don’t include all data values here, only relevant means, stdev etc. In most cases, all data points are not needed, especially for computer-based labs.

Table 1: Experimental data.

| Quantity name | Measured value, units |
| --- | --- |
| Incident angle, in degrees or radians | Points distance from 0-angle position, m |
| θ |  |

Provide description about each entry as needed, don’t just leave the table without text in this section. Organize well so that data takes less space, but everything needed is included.

## Theory

Consider the equation 1 that described the Snell’s law.

n1sinθ1=n2sinθ21

with n being the refractive indexes of media one and two, θ – angles to the normal to the interface. Always use equation editor and number the equations so that they later can be referenced as eq. 1 or equation 1. You will need many other equations and derivations in this section.

PHYS2212L – (required) include the derivation of Snell’s law in sufficient details. There are multiple derivations – use the one you like most and it is easiest one to understand.

## Analysis

Part 1

First, show that the glass walls don’t interfere with the measurement. Hint – write all interfaces as a single expression and see which n cancel!

Rewrite equation 1 so that if plotted, nwater is the slope. Propagate all errors to find error bars. For the sine of the angle in water, there is no need to find the angle – all we need is the sine and you have the sides of the triangle! This is easier in terms of error propagation. PHYS1112L can use simplified propagation equations, PHYS2212L as usual uses the derivative.

For the incident angle, there is no way to avoid the derivative use. Just recall that ∂∂θsinθΔθ=cosθ∙Δθwith the delta theta is radians!

Find the slope and compare with the accepted value for water refraction index within errors. (Hint – remember all other things to find as with any plot!)

## Conclusion

Comment on your result and the methods used to find it. What did you learn in performing this measurement? Where else can it be used? Can it be improved?

For PHYS1112L, another option is available. Remember from 1111L, there was a method 0.68∙sinθ+Δθ−sinθ−Δθ. Here, all angles and errors can be in degrees.