# Lab 10 manual: Index of refraction of water

Dr. D. Beznosko

4/9/2026

For PHYS 2212L only

(Lab outcomes: measuring angles and refraction index. Error propagation application to angles. Laser safety.)

## Abstract

The Snell’s law is used to measure the refraction index of water. Laser safety is explained, and goniometer is introduced. The angle errors are propagated through all equations.

## Introduction

The refraction n is the property of all materials, even non-transparent – in general, n is complex, and the imaginary part is the absorption, so non-transparent media will have only the imaginary part. Can n be less than 1 or negative? Can you find any more examples and practical uses?

## Experimental Setup

The experimental setup in Figure 1 shows the glass reservoir for water, the laser, and the goniometer. A piece of sticky tape is used for the screen. The tape is marked with the regular ball-pen.

Tape used as a screen

Goniometer

Laser

Water reservoir Glass walls

Light path without water

Light path with water

Distance between walls

Figure 1: The experimental setup schematic.

### Procedure

Listen to laser safety very carefully. Never look at the laser directly (the light emission point), never point it at any person or reflective surface!!

Fill the apparatus with water. Place the sticky tape on the reservoir wall. Do the measurements at 0o-60o. Mark all laser beam positions on the tape. Make sure that you aim the laser correctly (instructor will demonstrate). What is the error on θair? Measure distances of all points on the tape from the 0-angle point. What is the error on these measurements? Using caliper, measure the distance between the glass walls as indicated in Figure 1 (remove water first!). Consider this error as well.

## Experimental Data

The results of the measurements should be organized as Table 1 or similar. Think about how to organize this table. The error can be listed in a different column or as value ± error unit. Don’t include all data values here, only relevant means, stdev etc. In most cases, all data points are not needed, especially for computer-based labs.

Table 1: Experimental data.

| Quantity name | Measured value, units |
| --- | --- |
| Incident angle, in degrees or radians | Points distance from 0-angle position, m |
| θ |  |

Provide description about each entry as needed, don’t just leave the table without text in this section. Organize well so that data takes up less space, but everything needed is included.

## Theory

Consider equation 1 that described Snell’s law.

n1sinθ1=n2sinθ2                                                              (1)

with n being the refractive indexes of media one and two, θ – angles to the normal to the interface. Always use equation editor and number the equations so that they later can be referenced as eq. 1 or equation 1. You will need many other equations and derivations in this section, like the equation for the sinθ2 and all needed error propagations. Include the derivation of Snell’s law in sufficient details. There are multiple derivations – use the one you like most and it is easiest one to understand.

## Analysis

First, show that the glass walls don’t interfere with the measurement. Hint – write all interfaces as a single expression and see which n’s cancel!

Rewrite equation 1 so that when plotted, nwater is the slope. Propagate all errors to find error bars. For the sine of the angle in water, there is no need to find the angle – all we need is the sine and you have the sides of the triangle! This is easier in terms of error propagation.

For the incident angle, recall that ∂∂θsinθΔθ =cosθ∙Δθ with the delta theta is radians!

Find the slope and compare the result with the accepted value for water refraction index within errors. (Hint – remember all other things to find as with any plot!)

Use https://sos.clayton.edu/physics/chi2/ for any plotting.

## Conclusion

Comment on your result and the methods used to find it. What did you learn in performing this measurement? Where else can it be used? Can it be improved?