# Lab 5 manual: Linear combination of resistors

Dr. D. Beznosko

4/9/2026

For PHYS 2212L

(Lab outcomes: Solving a linear system of equations using computer; complexity reduction using different approaches.)

## Abstract

The measurement of even a simple quantity like resistance can be difficult at times as it’s not always possible to separate the parts of the circuit for this. The report describes the procedure of measuring the resistance of the inseparable circuit and suggests other possibilities for the measurements. Linear system of equations needs to be solved, and errors propagated.

## Introduction

The solutions for the systems of equations are often needed in numerous practical applications. The most common example of linear system solution is the eigenvalue problem that is widely used in physics, engineering, and other sciences. It is at the base of machine learning. Can you find any more examples and practical uses?

## Experimental Setup

R1

C

R2

R3

A

B

Multimeter

The experimental setup in Figure 1 shows the three resistors connected as a triangle. The resistance can be measured by the multimeter between the points A, B and C only.

Figure 1: The experimental setup schematic. A multimeter is used to measure the resistance between the points A, B and C.

### Procedure

Part 1

Using multimeter, measure RAB, RBC and RCA. Using the manual for the multimeter, get the accuracy and use it as systematic error. What is the statistical error? (hint – if the digits don’t change during the 5 measurements, use half of the smallest digit as a statistical error).

Write the expressions for RAB, RBC and RCA in terms of R1, R2 and R3. Can be left as reciprocal relationships, that is, 1RAB  ,1RBC and1RCA.

Part 2

Use the PASCO [1] computer interface or external power supply to apply voltage (about 3V) across the AB. Using PASCO interface and additional probe (or a multimeter), measure the voltage drop across R2 and R3. Derive that V2V3=R2R3  - show derivation in Theory section. Use the stdev (as statistical error), mean and move the window around 5 times for systematic error (measure 5 times using the multimeter). Repeat for BC and CA. Keep the data and all errors.

How does this simplify the equations from part 1? Write out the new equations. Find R1, R2 and R3 using these new equations and compare the results with part 1. Are the errors smaller or larger? Why so?

(Part 2.1, alternative. Do this part only if high precision current probe is available. Record the total current using additional probe (with both errors! Stdev as statistical and moving window as systematic). Using Ohm’s law and error propagation, are the RAB, RBC and RAC measured this way more accurate than from part 1? Find R1, R2 and R3 and compare to part 1)

Part 3

Using provided wire with alligator clips, short the AC, measure RAB; short the AB, measure RBC; short the BC, measure RCA. (Normally, you CAN NOT short anything in the circuit under voltage!!!) Use the best method to measure the resistance that you established in part 2 of this lab (multimeter or measuring voltage and current if additional current probe is available). Record the resistances and all errors. Write new expressions for RAB, RBC and RCA. Can be left as reciprocal relationships. Solve for R1, R2 and R3 and compare results with parts 1 and 2.

## Experimental Data

The results of the measurements should be organized as Table 1 or similar. Think about how to organize this table. The error can be listed in a different column or as value ± error unit. Don’t include all data values here, only relevant means, stdev etc. All data values must be included in the Appendix for this lab. In most cases, all data points are not needed, especially for computer-based labs.

Provide description about each entry as needed, don’t just leave the table without text in this section. Organize well so that data takes up less space, but everything needed is included.

Table 1: Experimental data.

| Quantity name | Measured value, units |
| --- | --- |
| RAB | Resistance, Ω |

## Theory

Consider the equation 1 that described the Ohm’s law.

V=IR                                                                             (1)

with I being the current, R - total resistance and V - total voltage across the elements. Always use equation editor and number the equations so that they later can be referenced as eq. 1 or equation 1. You will need many other equations and derivations in this section.

## Analysis

Part 1

Use the Wolfram Alpha [2] or similar computer software/online service to solve the system of equations for the R1, R2 and R3. Propagate errors and compare with the values of the resistors from the color marking. The marking has the %error, so each value is also with error. Do they match? You can use Mathematica syntax in Alpha as well.

Part 2

How can you use the ratios of resistances to make the equations from part 1 easier to solve? Suggest your solution (show the new equations) and find R1, R2 and R3. Compare, within errors, to two previous results and the values from the color codes. Comment on the comparison.

(If current probe was available: Redo part 1 calculations with the new values of RAB, RBC and RCA and compare within errors. Which measurement method is better, using multimeter or Ohm’s law? Why?)

Part 3

Again, solve the new system of three equations for RAB, RBC and RCA  to find R1, R2 and R3. Propagate errors and compare. Hint – finish all calculations and do the comparison of everything needed once in a separate section. Can the ratios of resistors from part 2 improve your result or make calculations simpler? Try to use them to solve for R1, R2 and R3 by suggesting your way to combine equations from part 2 and 3. Did the error grow or become smaller? Why?

For all parts, simplified error propagations can be used for this lab!

## Conclusion

Comment on your result (that is, R1, R2 and R3) and the methods used to find it. What did you learn in performing this measurement? Where else can it be used? Can it be improved?

# References

| [1] | PASCO Scientific, 10101 Foothills Boulevard, Roseville, California 95747 USA. |
| --- | --- |
| [2] | "Wolfram Alpha," Wolrfam Inc., [Online]. Available: https://www.wolframalpha.com/. |