# Lab 8 manual: em measurement (Tracker)

Dr. D. Beznosko

3/21/2025

For PHYS 2212L only

(Lab outcomes: particle deviation in magnetic field, using Tracker for radius measurements.)

## Abstract

The objective of this laboratory is to measure the charge to mass ratio of an electron. The video of the experiment is used for this purpose. The result is compared with the accepted value within errors. This lab also teaches to accept that experiments don’t always work out the way you expect them to.

## Introduction

The first person to measure this charge to mass ratio for the electrons was J. J. Thomson in 1897 for what was called at the time ‘cathode rays’. Later this experiment result convinced him that these rays were particles with the negative change. Now we call them electrons. Remember to reference historic information as well! What do we use electrons for today? Can we use them to power cars and from which source?

## Experimental Setup

e/m apparatus

Ruler

Multimeters

Multimeters insert

Power supply

Shown in Figure 1, the experimental setup that consists of the ruler, two multimeters, e/m apparatus, and power supplies. Provide DETAILED description of PASCO [1] e/m apparatus!

Figure 1: The experimental setup schematic. An insert in the right lower corner shows the top view on both multimeters.

### Procedure

You can use the individual photos provided or the video that combines all five photos as individual frames for convenience (recommended). For the frame/photo 1, set the scale by using a ruler and use the Circle Finder tool to measure the radius of the circle that is drawn by a bright blue filament in the bulb. Choose the approximate center of the filament, when possible, but stay towards the inner radius, ignore the outside glowing halo. Holding shift, choose three points around the circle (see video tutorial if needed at https://youtu.be/IrD10scYpsA?t=2932) and record the radius. Do this at least 5 times by deleting the circle and adding it back. Find average of the radius and the stdev – this is your statistical error. Keep the last circle fitter result and delete and add the ruler 5 times – the stdev of the radius will be the systematic error. Combine them in quadrature. You can use the same systematic error for all 5 frames/photos. Record the current and accelerating voltage from the multimeters, and their errors (use precision as systematic. Find it in the multimeter manual. Show details for finding all errors for at least 1 frame!).

Measure radius and record the voltage from the remaining four frames/photos. With errors!

## Experimental Data

The results of the measurements should be organized as Table 1 or similar. Think about how to organize this table. The error can be listed in a different column or as value ± error unit. Don’t include all data values here, only relevant means, stdev etc. What are the voltage and current errors?

Table 1: Experimental data.

| Accelerating Voltage, V | Radius, m | Current, A |
| --- | --- | --- |

Provide description about each entry as needed in the section text, don’t just leave the table without text in this section. Organize well so that data takes up less space, but everything needed is included.

## Theory

As a charged particle enters the magnetic field directed perpendicularly to the velocity of the particle, it starts to rotate with a radius of orbit

r=mvqB1

where m is the mass of the particle, v is the speed of the particle, q is the magnitude of the particle's charge, and B is the strength of the magnetic field. For an electron, the equation 1 becomes

r=meveB2

where e is the magnitude of the charge of an electron and me is electron’s mass.

The magnetic field produced near the axis of a pair of Helmholtz coils is given by the equation:

B=4532Nμ0Ia3

where N is the number of turns on each Helmholtz coil (N=130 in our case), μ0 is the magnetic permeability of free space, and a is the radius of the Helmholtz coils (a=150mm).

After the electrons are accelerated through the accelerating potential difference V, gaining kinetic energy, their speed becomes:

v=2eVme4

Combining equations (2), (3), and (4) produces the final formula for e/m ratio:

eme=5432Va2Nμ0Ir25

Show this derivation of equations 1, 4, 3 and 5, filling in missing steps.

Clearly state what you are going to plot on x and on y axes – the slope should be the e/m ratio! Propagate the error for y-values and x-values, show equations and details!

## Analysis

First, show what are you going to plot – you want you slope to equal to em, the value that you seek. For other values, check out https://cdn.pasco.com/product_document/012-14265C.pdf or google for the PASCO e over m apparatus and read the documentation. While the manual gives N and a (for the manual, a=R), they don’t give errors. Assume some error for N and a – what do you think is the error for these values? The average radius a is the center of the coils, what is the reasonable error to assume? For N, may be ±1 turn or just use as perfect? You may use μ0 as perfect – check out the error on the known value, its tiny compared to other error contributions in our experiment. Use https://sos.clayton.edu/physics/chi2/ for any plotting. Plot the values that you found, find the slope that is the e/m ratio, with slope error and χ2. Compare your result with the accepted value within the errors.

## Conclusion

Comment on your result and the methods used to find it. What did you learn in performing this measurement? Where else can it be used? Can this experiment be improved? Can you trace the problem to its source?

# References

| [1] | PASCO Scientific, 10101 Foothills Boulevard, Roseville, California 95747 USA. |
| --- | --- |

For the video, an older setup was used, so use a=150mm